Water Wash Batch (WWB) Presentation

#1

EXCO

WWB Treatment in the Crude EXCO Flow ™ technology

By:

Hector Cruz Cueto, Engineer

2019

#2

WWB Crude Wash

- This technology was developed due to a persistent issue in the PEMEX system.

- The salt content in the crude is considerably high when it arrives at the refinery (150-400 lb/1000 bbl).

- With current sedimentation time, it is extremely hard to reach values of 5 lb/1000 bbl to be able to process product in the desalination.

- In addition to the emulsion’s formation with congenital water and cleaning water in the desalination.

- All these issues cause corrosion, soiling, high pressure in heaters, and loss of operation efficiency.

- It also presents formation of ammonium chloride and iron problems which will initiate serious soiling and corrosion issues in hydrodesulfurization plant, FCC, cookers, and heaters.

#3

WWBS Crude Wash

- The application of this treatment is necessary upon the arrival of crude oil to any refinery or distribution center.

- In preparation for the treatment the following conditions are required:

    - Conditioning of water wash (fire retardant) using compound chemicals from EXCO Flow ™

    - The salt content (lb/1000 bbls) in the crude oil, will determine the amount of water wash for conditioning.

    - Based on the infrastructure (already installed) inject the solution for a better and more efficient mixing.

    - Let it stand in standing tanks for 7 – 12 hours.

    - Drain remaining water and after verifying resulting salt values, if results are satisfying, send to stabilizing tanks.

- With this WWBS Crude Washing technology, it is possible to lower the salt content up to a 70% less of its original arriving value.

#4

Maya crude’s pipeline testing

- Feasibility testing:

    - Asphaltene: 4 – 8% unstable

    - Paraffines: 13 – 15%

    - Water and sediments: 0.5 – 3%, 0.1 – 1%

    - Salt: 150 – 400 lb/1000 bbls

    - Acidity: 3.6 -4.3%

    - Emulsion: very consistent (it contains agglomerates and added dispersants in wells)

#5

Tanks TV-2005 and TV-2006

- Maya crude’s receptor tanks testing:

TV-2005		TV-2006

Asphaltene:			   12.96			   11.86

Paraffines:			     9.75			   10.92

Water and Sediments:	           T:0.5, M:1.2 F:24	         T:0.2, M:0.5 F:26

Salt PTB:			 267-280		 189-242

Acidity:				      3.89	    		     3.42

Emulsion:			Very Persistent	            Very Persistent

- The tank drainage deteriorates due to the sediment accumulation and the crude water persistent emulsion in the tanks.

#6

EXCO Flow ™ Treatment

- Based on the previous data, treatment was added to arriving crude line, tank was drained, and we obtained the following results.

RESTING TIME

Salt PTB		7 hours			11 hours		18 hours

74			     26		                   11.6                                    9.2

68		                    22		                    10.2	                                  9.5

90		                   22.3		                    13.7	                                 11.4

- Based on these results, we can resolve the emulsion problem and remove salt in crude up to a 74.9% in 7 hours.

#7

Salt behavior in Maya crude as it arrives at Minatitlan refinery

Lab evaluation with EXCO Flow ™ Treatment

Salt in Maya crude

Water and sediments in Maya crude

#8

WWB process flow diagram

(Insert diagram)

#9

Experimental test endorsed by Crude Wash EXCO Flow ™

#10

Technical Proposal

- Our evaluation proposal is focused to better your operation conditions

- We propose 8 days of our WWBS in crude

- All results will always be overseen and approved by the operating personnel

- We propose to draw initial samples from arriving crude and storage tanks to determine % of water, sediment, and salt PTB with base line

- Subsequently, we will be taking samples every hour and will register amount of water drained from tank to establish a balance between congenital water, the water wash, and drained water itself

- We will provide daily reports and at the end of treatment

- We will keep all safety and environmental standards

#11

INJECTOR INSTALLATION TO ARRIVING CRUDE PIPELINE